Array Tool 3C: Documentation

1. How to prepare the analysis and run the program

Copy array_tool_3c.tar.gz to your working-directory.

←	configuration-files

adapt sac.list before running the program

←	routines used by the main program

←	output-files

out.m3c = output-file from ArrayTool3C

m3c_convert generates input-files 	(output.*.max) for maxtocurve (Geopsy)

←	data in sac-format

*.1.sac = vertical components

*.2.sac = NS components

*.3.sac = EW components

←	main program

To prepare your analysis, put all the data you want to use in a folder called sac. The vertical components need to have the ending *.1.sac, the NS components *.2.sac and the EW components *.3.sac. When you are using Swiss Coordinates they have to be defined in the sac-header at position USER7 and USER8, Lon/Lan at STLO and STLA. Please check the routine import_record (in the folder mfiles) if the right header-definition is activated.

Adapt the highlighted parts of sac.list to your data. Lines starting with a # are not read by the program, they are just comments.

# -----------------------------------------

# ARRAY TOOL 3C - FILE LIST

# -----------------------------------------

# NAME = M2.1

# LOCATION = ETH Zurich

# DATE = ../../..

# -----------------------------------------

# Root directory

ROOT sac/

# SAC files: Channel1 Channel2 Channel3

SACF M02.1.3001_0491_0512_0000.1.sac M02.1.3001_0491_0512_0000.2.sac M02.1.3001_0491_0512_0000.3.sac

SACF M02.1.3001_0500_0507_0000.1.sac M02.1.3001_0500_0507_0000.2.sac M02.1.3001_0500_0507_0000.3.sac

...

Open matlab, go to your working-directory and type in array_tool_3c for starting the program. If you want to run the program without a graphical interface (faster!), use the command array_tool_3c ('cfg/param.cfg'). But in this case you have to define the parameters explained in the next chapter in advance in the file param.cfg.

2. Setting the parameters

Input / Output

Select the sac-file list (*.list) you have prepared. All the noise-data is then shown in the Trace plot. To have a look at the different channels, enable Ch.1 (vertical), Ch.2 (NS) or Ch.3 (EW) and press the Redraw-button. The sensor-geometry can be checked in the XY plot. If everything is fine with your data, define the destination-folder and the name for the output file (*.m3c).

Frequency / Velocity

Choose the minimum and maximum values of the frequency- and velocity-range you want to analyse as well as the resolution (number of values between). The spacing can be logarithmic or linear.

Azimuth

For the analysis, the information of the horizontal wavefield from N different directions is used. If you set N=180, the f-k-Spectra is calculated at each 1° (180°/N).

Core Algorithm

You can choose between four different algorithms.

- Classic FK: normal beamforming

- Capon (D.L.): high resolution beamforming

- Capon (S.V.D): Based on the assumption that an amplitude maximum in the f-k cross-spectrum must represent the true power amplitude of the corresponding signal, in addition to the Dispersioncurves, also the Rayleigh wave ellipticity is retrieved using high-resolution f-k array analysis. (Poggi, 2011)

- MUSIC: selection of the strongest signal! You may lose information about higher modes.

Windowing

Choose between "Fixed Length" (define the length of a window in seconds) or "Frequency Depended" (define the number of cycles). The taper can be between 0 (boxcar function) and 1 (sinusoidal).

Figure: Taper-functions for different values (Valerio)

Frequency Averaging

Instead of making a kx-ky plot for one exact value fi, you can choose a number of f-values inside a selected interval (e.g. [fi-1+(fi-fi-1)/2   fi+(fi+1-fi)/2] if the interval is defined as 100%, see picture below), calculate kx-ky for each of this f-values and then take the average as representative  for fi.

100%                                                                      50%

fi-1                   fi                      fi+1

fi-1                    fi                      fi+1

C-Matrix Stacking

The f-k-analysis is not made for each single window (the shape of this windows was defined before in "Windowing"), but for the summation of N windows. You also have to choose how much the separate windows should overlap (0% = no overlap, 100% = total overlap).

Rule of thumb for optimal window overlapping percentage:

overlap = taper · 50 		                  (taper [0 1] was chosen in "Windowing")

Figure: Different plots that show how the result changes as function of #Windows and Overlap. (Clotaire)

Maxima search

Here you can define the number of maximas you want to have picked in the kx-ky plot (for example # = 5 in the picture below). If the "Resolution Filter" is activated, the maxima search only takes place inside the wave-number resolution limits kmin=12Dmax (Dmax = maximum existing distance between sensors) and kmax=12Dmin (Dmin = minimum distance between sensors). If you choose # = 1 you will only pick the most energetic mode, mostly the fundamental one. But if the number of selected maxima is too large it will be difficult to distinguish between real higher modes and artifacts. There is no rule of thumb which number of maximas to use, this has to be tested from case to case.

3. Output

The output-file is called *.m3c. In the header you can find the parameters used for the analysis again. This makes it easy to assign all your output-files to different runs with varying parameter-settings. For all four Core Algorithms you will get the same type of output (see below), but only for "Classic FK" and "Capon (S.V.D)" it makes sense to use the ellipticity.

1. column	2. column	3. column	4. column	5. column	6. column

component	frequency	velocity		azimuth		powerspectra	ellipticity

With the command m3c_convert *.m3c you will produce three different files (vertical, radial and transversal component) that can be used as input for the max2curve-tool of Geopsy.

3.1 Output file

3.2 Azimuth selection

With the command azimuth_filter.m you can produce a new .m3c file including a selection of azimuth only. Azimuth is here considered as the angle from the E direction (X) counted anti-clockwise. The values should be given between 0 and 360.

4. F-k Analysis: Short Introduction (after Tokimatsu, 1997)

The phase velocity of Rayleigh- and Love waves can be determined through the frequency-wavenumber (f-k) spectrum analysis developed by Capon (1969). The wavefield is described as a spectral f-k density-function. This function describes the energy of a wave according to the frequency and wave velocity. Assuming that M sensors are available and that the ith sensor is located at the vector position xi, the f-k spectrum, P(f,k), is defined by

P(f,k) = i,j=1MAi*(f,k)Aj(f,k)Gij(f)exp[ik·(xi-xj)]

where * denotes complex conjugate, f is the frequency and k is the vector wavenumber in cycles per meter defined as

k = |k| exp(iθ)

in which θ is the azimuth of the vector wavenumber, measured clockwise from the x-axis as shown in Fig. 1.

Figure 1: Schematic diagram for microtremor measurements (Tokimatsu, 1997).

For the conventional method (Classic FK) Ai(f,k) is defined as Ai=1, and for the high resolution method (Capon)

Ai(f,k) = j=1Mqij(f,k) / i,j=1Mqij(f,k)

in which {qij(f,k)} is the inverse of the matrix {exp[ik·(xi-xj)]Gij(f)}. Gij(f) is the cross power spectrum between the ith and jth sensors at a frequency f, and is (based on the direct segment method after Capon (1969)), determined by

Gij(f) = 1Nn=1NSin(f)Sjn*(f)

in which N is the total number of the nonoverlapping data segments, and Sin is the Fourier transform of the data in the ith sensor and in the nth segment. The f-k spectrum is drawn on a two-dimensional wavenumber (kx-ky) space for each frequency, in which the positive ky-axis points to the north (Fig. 2). The peak of this f-k spectrum provides the information concerning the phase velocity and the azimuth of the source. If a peak occurs at a distance of |kp| from the origin at a frequency f, the corresponding phase velocity, c, and the wavelength, λ, can be given by

c = 2πf / |kp|

λ = 2π / |kp|.

By repeating this computation over the frequency range for all data from different array diameters, a dispersion curve can be obtained.

Figure 2: Example of f-k spectra and corresponding dispersion data (Tokimatsu, 1997).

5. Literature

Capon, J., 1969. High-Resolution Frequency-Wavenumber Spectrum Analysis, Proceedings of the IEEE, 57, 1408-1418.

Fäh, D., Stamm, G. and H.-B. Havenith, 2008. Analysis of three-component ambient vibration array measurements, Geophys. J. Int., 172, 199-213.

Poggi, V., 2011. The use of surface waves for site characterization and seismic hazard analysis, Dissertation No. 19534, ETH Zürich.

Tokimatsu, K., 1997. Geotechnical site characterization using surface waves, Proc. 1st Intl. Conf. Earthquake Geotechnical Engineering, 1333-1368.

6. Possible error-messages and how to handle them

While loading the input-file sac.list:

Error using ==> rsac at 90

LEVEN must = 1; SAC file not evenly spaced

Known problem of routine rsac.m. Open the files with sac2000 or Geopsy and resave them. Now it should work...